Date: 26-04-2019

Summary Report on WIT & WIL

(Daily Report)

| Name of the Faculty: Dr. T. Lakshmi Viveka |  |  | Name of Subject: Engineering Chemistry |
| --- | --- | --- | --- |
| Class/Section: AE |  |  |  |
|  | Grid Reference No.: | 3.3( REVISION) |  |  |
|  | Scenario Reference No. (Mapping with Syllabus) |  |  |  |
|  | Topic covered in every class | Batteries and fuel cells |  |  |
|  | Brief write-up (500 words) for every class: | Lithium battery A lithium-ion battery or Li-ion battery (abbreviated as LIB) is a type of rechargeable battery in which lithium ions move from the negative electrode to the positive electrode during discharge and back when charging. Li-ion batteries use an intercalated lithium compound as one electrode material, compared to the metallic lithium used in a non-rechargeable lithium battery. Lithium-ion batteries are common rechargeable batteries for portable electronics, with a high energy density, no memory effect (other than LFP cells) and low self-discharge Pure lithium is highly reactive. It reacts vigorously with water to form lithium hydroxide (LiOH) and hydrogen gas. Thus, a non-aqueous electrolyte is typically used, and a sealed container rigidly excludes moisture from the battery pack. Lithium-ion batteries are more expensive than NiCd batteries but operate over a wider temperature range with higher energy densities. They require a protective circuit to limit peak voltage. Fuel cell Direct-methanol fuel cells or DMFCs are a subcategory of proton-exchange fuel cells in which methanol is used as the fuel. Their main advantage is the ease of transport of methanol, an energy-dense yet reasonably stable liquid at all environmental conditions. Efficiency is quite low for these cells, so they are targeted especially to portable applications, where energy and power density are more important than efficiency. A more efficient version of a direct fuel cell would play a key role in the theoretical use of methanol as a general energy transport medium, in the hypothesized methanol economy. |  |  |
|  | Relevant additional illustration if any: | LIBs are also growing in popularity for military, battery electric vehicle and aerospace applications. |  |  |
|  | Video Links/ Web Links if any: | https://www.youtube.com/watch?v=kqR7MihP5k4&t=2s https://www.youtube.com/watch?v=6oeN9VDFLig |  |  |
|  | Signature of Repository Administrator: |  |  |  |